ASSEMBLY AWARE TEXT EDITOR

1. ABSTRACT

The Assembly Aware Text Editor is a specialized Integrated Development Environment (IDE) designed for 8086 Assembly language programming. Unlike conventional editors, it provides real-time syntax and semantic analysis while the user types, allowing immediate detection of errors such as invalid instructions, incorrect operands, and addressing mode issues.

The system enhances code readability through syntax highlighting and reduces debugging time by identifying errors early. It also supports optional integration with EMU8086 for assembling and executing programs. This project aims to improve learning efficiency for students and beginners in low-level programming.

2. OBJECTIVES

·  Design a real-time Assembly code editor

·  Implement syntax and semantic error detection

·  Provide 8086-specific instruction validation

·  Develop a user-friendly interface using Python (Tkinter)

·  Integrate with EMU8086 for execution

·  Enhance learning experience for beginners

3. SCOPE

Included:

- 8086 Assembly language support

- Real-time syntax highlighting

- Error detection and validation

- Automatic saving mechanism

- EMU8086 integration

Limitations:

- Supports only 8086 architecture

- Limited macro and advanced instruction support

- No optimization or compiler-level analysis

4. USERS

1. Students (Primary Users)

- Engineering students learning Assembly Language

- Use the editor to write, test, and debug programs

- Benefit from syntax highlighting and error detection

- Helps understand registers, memory addressing, and instruction flow

- Reduces confusion compared to plain text editors

2. Beginners in Low-Level Programming

- New learners with no prior assembly experience

- Use features like:

  - Instruction reference

  - Error panel

  - Memory validation

- Helps avoid common mistakes (syntax errors, invalid addresses)

- Acts as a guided learning tool

3. Embedded Systems Learners

- Students working on microcontrollers and hardware-level programming

- Use the editor to simulate and validate assembly code

- Helps understand:

  - Memory mapping

  - Register usage

  - Instruction execution flow

5. SOFTWARE INTEGRATION

The Virtual CPU Scheduler Simulator integrates the following software components and environments:

| Category | Details |
| --- | --- |
| EMU8086 Emulator | Core runtime environment for executing x86 assembly code. Provides a virtual 8086 CPU, memory model, register panel, and I/O simulation. All assembly modules are assembled and run within this environment. |
| x86 Assembly Language | The primary programming language used for all simulation logic — scheduling algorithms, data storage, calculations, I/O, and display. Uses the 8086 instruction set (MOV, CMP, JMP, LOOP, INT, etc.). |
| BIOS Interrupts (INT 10h) | Used for low-level screen control — setting cursor position, clearing screen regions, and applying text color attributes for enhanced visual output. |
| Windows OS | Host operating system required to run the EMU8086 emulator. The simulator is platform-dependent on Windows due to EMU8086 compatibility. |

6. HARDWARE ENVIRONMENT

| Category | Details |
| --- | --- |
| Laptop | Processor : Intel i5-12500H Storage : 512 GigaBytes Ram : 16 GB DDR5 Graphics : RTX 3050 |

7. BLOCK DIAGRAM WITH DESCRIPTION

Block Diagram Description

User UI (Editor Window)

This is where the user writes and interacts with code. The process here is capturing keystrokes, edits, and commands, then immediately forwarding them to the controller. It acts as a real-time input/output surface, reflecting processed results like highlights and errors.

editor_app.py (Main Controller)

This is the brain that coordinates everything. The process involves receiving user input, routing it to appropriate modules (logic, visualization, execution), and synchronizing outputs back to the UI. It ensures all components work together without conflict.

Input Logic

This block handles correctness and structure of code. The process is continuous analysis of user input—checking syntax, validating addresses, and enforcing rules before execution or display updates.

- syntax_highlighter.py
The process scans code in real-time, identifies tokens (keywords, variables), and applies visual styles instantly. It improves readability without altering logic.

- linter.py
The process analyzes code for errors, warnings, and bad practices. It flags issues early, preventing faulty execution.

- addr_validator.py
The process validates memory addresses or references, ensuring they are valid and safe before execution. This prevents runtime faults.

Visualization Components

This block improves how information is presented. The process converts raw analysis into visual feedback for the user.

- line_numbers.py
The process dynamically tracks and updates line indices as code changes, maintaining alignment with the editor.

- error_panel.py
The process collects errors from the linter and displays them in a structured format, allowing quick debugging.

Background Tasks

This runs silently without interrupting the user. The process is periodic execution of supportive tasks.

- autosave.py
The process continuously monitors changes and saves the file at intervals, preventing data loss without user action.

emu_bridge.py (Execution Layer)

This is the bridge between editor and execution engine. The process translates validated code into a format suitable for execution and sends it to the ASM engine, then retrieves results back.

ASM Engine / Files (.ASM modules)

This is where actual execution happens. The process involves assembling and running low-level instructions, then returning output or errors back through the bridge to the UI.

8. MODULE DIAGRAM WITH DESCRIPTION

The module diagram below presents a structured view of all ten software modules that constitute the Virtual CPU Scheduler Simulator, along with their individual responsibilities and interactions.

Module Descriptions

main.py (Application Entry Point)
This module initializes the entire system, loads configurations, and starts the main execution flow. It acts as the starting controller that ensures all components are properly triggered.

core_app.py (Main Version)
This is the central controller that manages workflow and data flow between all modules. It coordinates communication between Python modules and invokes ASM functions where required.

autoave.py (Online Guided Save)
This module continuously monitors user progress and automatically saves results to prevent data loss. It ensures recovery support and maintains session continuity.

interaction_ref.py (Frame Tester)
Handles user interactions and acts as a bridge between the user interface and backend logic. It sends inputs to the core system and processes feedback responses.

emu_bridge.py (EMU8086 Bridge)
This module connects the application with the EMU8086 emulator to run assembly code. It manages execution, transfers inputs, and retrieves results for further processing.

error_panel.py (Error Panel)
Responsible for detecting, displaying, and logging errors generated during execution. It also suggests possible fixes to help users debug efficiently.

live_numbers.py (Live Numbers)
Performs real-time calculations and updates dynamic data during execution. It enhances visualization by reflecting changes instantly in the system.

links.py (ASM Index)
Maintains a structured list of all assembly modules and their functionalities. It provides quick navigation and access to different ASM components.

syntax_highlighter.py (Code Highlighter)
Improves code readability by highlighting assembly syntax and structure. It also supports themes and helps users better understand code formatting.

Data Handling & Config (JSON, TXT, Directories)
Manages all configurations, file paths, and stored data required by the system. It ensures smooth loading of test cases and maintains user-specific settings.

addr_validator.py (Memory Address Check)
Validates memory addresses used in assembly programs to prevent runtime errors. It checks for alignment and range correctness before execution.

asm_keywords.py (Mnemonics, Registers, Directives)
Stores all assembly-related keywords and definitions required for parsing and validation. It supports both syntax checking and code understanding within the system.

9. SOURCE CODE

MODULE 1 - Syntax Highlighter :

.MODEL SMALL

.CODE

ORG 100h

MAIN_LOOP:

MOV  DX, OFFSET MSG_PROMPT

CALL PRINT_STR

MOV  AH, 0Ah

MOV  DX, OFFSET INPUT_BUF

INT  21h

MOV  DX, OFFSET CRLF

CALL PRINT_STR

MOV  SI, OFFSET INPUT_BUF + 1

XOR  CH, CH

MOV  CL, [SI]

INC  SI

MOV  DI, OFFSET WORD_BUF

MOV  [WORD_LEN], CL

JCXZ MAIN_LOOP

TOUPPER_LOOP:

MOV  AL, [SI]

CMP  AL, 61h

JB   STORE_C

CMP  AL, 7Ah

JA   STORE_C

SUB  AL, 20h

STORE_C:

MOV  [DI], AL

INC  SI

INC  DI

LOOP TOUPPER_LOOP

MOV  BYTE PTR [DI], 0

MOV  SI, OFFSET WORD_BUF

MOV  DI, OFFSET KW_EXIT

CALL STR_CMP

JZ   END_PROGRAM

CALL CLASSIFY_WORD

JMP  MAIN_LOOP

END_PROGRAM:

MOV  DX, OFFSET MSG_BYE

CALL PRINT_STR

MOV  AH, 4Ch

INT  21h

CLASSIFY_WORD PROC

MOV  SI, OFFSET MNEMONIC_TABLE

CALL SEARCH_TABLE

JZ   IS_MNE

MOV  SI, OFFSET REGISTER_TABLE

CALL SEARCH_TABLE

JZ   IS_REG

MOV  SI, OFFSET DIRECTIVE_TABLE

CALL SEARCH_TABLE

JZ   IS_DIR

MOV  DX, OFFSET MSG_UNKNOWN

CALL PRINT_STR

MOV  DX, OFFSET WORD_BUF

CALL PRINT_STR

MOV  DX, OFFSET TAIL_UNK

CALL PRINT_STR

RET

IS_MNE:

MOV  DX, OFFSET MSG_MNEMONIC

CALL PRINT_STR

MOV  DX, OFFSET WORD_BUF

CALL PRINT_STR

MOV  DX, OFFSET TAIL_MNE

CALL PRINT_STR

RET

IS_REG:

MOV  DX, OFFSET MSG_REGISTER

CALL PRINT_STR

MOV  DX, OFFSET WORD_BUF

CALL PRINT_STR

MOV  DX, OFFSET TAIL_REG

CALL PRINT_STR

RET

IS_DIR:

MOV  DX, OFFSET MSG_DIRECTIVE

CALL PRINT_STR

MOV  DX, OFFSET WORD_BUF

CALL PRINT_STR

MOV  DX, OFFSET TAIL_DIR

CALL PRINT_STR

RET

CLASSIFY_WORD ENDP

SEARCH_TABLE PROC

NXT:

MOV  AL, [SI]

CMP  AL, 0FFh

JE   NOT_FND

XOR  CH, CH

MOV  CL, AL

CMP  CL, [WORD_LEN]

JNE  SKIP

INC  SI

MOV  DI, OFFSET WORD_BUF

PUSH CX

PUSH SI

CMP_CH:

MOV  AL, [SI]

CMP  AL, [DI]

JNE  MISM

INC  SI

INC  DI

LOOP CMP_CH

POP  SI

POP  CX

XOR  AX, AX

RET

MISM:

POP  SI

POP  CX

DEC  SI

SKIP:

XOR  CH, CH

MOV  CL, [SI]

INC  CX

ADD  SI, CX

JMP  NXT

NOT_FND:

OR   AX, 1

RET

SEARCH_TABLE ENDP

STR_CMP PROC

SL:

MOV  AL, [SI]

MOV  BL, [DI]

CMP  AL, BL

JNE  SN

CMP  AL, 0

JE   SE

INC  SI

INC  DI

JMP  SL

SE:

XOR  AX, AX

RET

SN:

OR   AX, 1

RET

STR_CMP ENDP

PRINT_STR PROC

MOV  AH, 09h

INT  21h

RET

PRINT_STR ENDP

INPUT_BUF   DB  20, 0, 21 DUP(0)

WORD_BUF    DB  21 DUP(0)

WORD_LEN    DB  0

KW_EXIT     DB  'EXIT', 0

MSG_PROMPT  DB  0Dh,0Ah,'  Input   : ','$'

CRLF        DB  0Dh,0Ah,'$'

MSG_MNEMONIC  DB '  Result  : [MNEMONIC]  --> ','$'

MSG_REGISTER  DB '  Result  : [REGISTER]  --> ','$'

MSG_DIRECTIVE DB '  Result  : [DIRECTIVE] --> ','$'

MSG_UNKNOWN   DB '  Result  : [UNKNOWN]   --> ','$'

TAIL_MNE    DB ' is a valid 8086 instruction opcode.',0Dh,0Ah,'$'

TAIL_REG    DB ' is a valid 8086 register name.',0Dh,0Ah,'$'

TAIL_DIR    DB ' is a valid assembler directive.',0Dh,0Ah,'$'

TAIL_UNK    DB ' is NOT a recognised 8086 keyword.',0Dh,0Ah,'$'

MSG_BYE     DB  0Dh,0Ah,' Process_Finished',0Dh,0Ah,'$'

MNEMONIC_TABLE:

DB 3,'MOV', 4,'PUSH',3,'POP', 3,'ADD',3,'SUB',3,'MUL',3,'DIV'

DB 4,'IDIV',4,'IMUL',3,'AND',2,'OR', 3,'XOR',3,'NOT',3,'NEG'

DB 3,'CMP',4,'TEST',3,'JMP',2,'JE', 3,'JNE',2,'JZ', 3,'JNZ'

DB 2,'JA', 3,'JAE',2,'JB', 3,'JBE',2,'JG', 3,'JGE',2,'JL',3,'JLE'

DB 2,'JS', 3,'JNS',2,'JO', 3,'JNO',4,'JCXZ',4,'CALL',3,'RET'

DB 4,'RETF',4,'LOOP',5,'LOOPE',5,'LOOPZ',6,'LOOPNE',6,'LOOPNZ'

DB 3,'NOP',3,'HLT',3,'INT',4,'INTO',4,'IRET',2,'IN',3,'OUT'

DB 3,'LEA',3,'LDS',3,'LES',4,'XCHG',4,'XLAT',3,'CBW',3,'CWD'

DB 3,'SHL',3,'SHR',3,'SAL',3,'SAR',3,'ROL',3,'ROR',3,'RCL',3,'RCR'

DB 4,'MOVS',5,'MOVSB',5,'MOVSW',4,'CMPS',5,'CMPSB',5,'CMPSW'

DB 4,'SCAS',5,'SCASB',5,'SCASW',4,'LODS',5,'LODSB',5,'LODSW'

DB 4,'STOS',5,'STOSB',5,'STOSW',3,'REP',4,'REPE',4,'REPZ'

DB 5,'REPNE',5,'REPNZ',3,'STC',3,'CLC',3,'CMC',3,'STD',3,'CLD'

DB 3,'STI',3,'CLI',5,'PUSHF',4,'POPF',3,'AAA',3,'AAD',3,'AAM'

DB 3,'AAS',3,'DAA',3,'DAS',5,'BOUND'

DB 0FFh

REGISTER_TABLE:

DB 2,'AX',2,'BX',2,'CX',2,'DX',2,'AH',2,'AL',2,'BH',2,'BL'

DB 2,'CH',2,'CL',2,'DH',2,'DL',2,'SP',2,'BP',2,'SI',2,'DI'

DB 2,'IP',2,'CS',2,'DS',2,'SS',2,'ES'

DB 0FFh

DIRECTIVE_TABLE:

DB 7,'SEGMENT',4,'ENDS',4,'PROC',4,'ENDP',2,'DB',2,'DW',2,'DD'

DB 3,'EQU',6,'ASSUME',3,'END',5,'MODEL',5,'SMALL',5,'LARGE'

DB 6,'MEDIUM',4,'CODE',4,'DATA',5,'STACK',4,'NEAR',3,'FAR'

DB 3,'ORG',6,'OFFSET',3,'PTR',4,'BYTE',4,'WORD',5,'DWORD'

DB 5,'LABEL'

DB 0FFh

END MAIN_LOOP

Module 2 : Address Validator

.MODEL SMALL

.CODE

ORG 100h

START:

LOOP_TOP:

MOV  DX, OFFSET MSG_PROMPT

CALL PRINT_STR

MOV  AH, 0Ah

MOV  DX, OFFSET INPUT_BUF

INT  21h

MOV  DX, OFFSET CRLF

CALL PRINT_STR

MOV  SI, OFFSET INPUT_BUF+1

XOR  CH, CH

MOV  CL, [SI]

INC  SI

MOV  DI, OFFSET EXPR_BUF

JCXZ LOOP_TOP

UP_LOOP:

MOV  AL, [SI]

CMP  AL, 'a'

JB   UC_OK

CMP  AL, 'z'

JA   UC_OK

SUB  AL, 32

UC_OK:

MOV  [DI], AL

INC  SI

INC  DI

LOOP UP_LOOP                    ; Repeat for all characters

MOV  BYTE PTR [DI], 0           ; Null-terminate EXPR_BUF

MOV  SI, OFFSET EXPR_BUF

MOV  DI, OFFSET KW_EXIT

CALL STR_CMP                    ; Compare EXPR_BUF with "EXIT"

JZ   QUIT

CALL VALIDATE_ADDR

JMP  LOOP_TOP

QUIT:

MOV  DX, OFFSET MSG_BYE

CALL PRINT_STR

MOV  AH, 4Ch

INT  21h

VALIDATE_ADDR PROC

MOV  SI, OFFSET EXPR_BUF

MOV  AL, [SI]

CMP  AL, '['

JNE  NOT_MEM                ; Must start with '[' to be a memory reference

MOV  DI, SI

FIND_CLOSE:

MOV  AL, [DI]

CMP  AL, 0

JE   NO_CLOSE               ; Reached end of string without finding ']'

CMP  AL, ']'

JE   HAS_CLOSE              ; Found closing bracket

INC  DI

JMP  FIND_CLOSE

NOT_MEM:

MOV  DX, OFFSET MSG_INVALID ; Not a memory expression

CALL PRINT_STR

RET

NO_CLOSE:

MOV  DX, OFFSET MSG_INVALID ; Missing closing ']'

CALL PRINT_STR

RET

HAS_CLOSE:

; -- Extract content between '[' and ']' into INNER_BUF --

MOV  SI, OFFSET EXPR_BUF

INC  SI                     ; Skip opening '['

MOV  DI, OFFSET INNER_BUF

COPY_INNER:

MOV  AL, [SI]

CMP  AL, ']'

JE   INNER_DONE             ; Stop at closing ']'

MOV  [DI], AL

INC  SI

INC  DI

JMP  COPY_INNER

INNER_DONE:

MOV  BYTE PTR [DI], 0       ; Null-terminate INNER_BUF

MOV  SI, OFFSET INNER_BUF   ; SI now points to inner content

MOV  DI, OFFSET REG_BX

CALL CHECK_PREFIX           ; Check for "BX"

JZ   BASE_OK

MOV  DI, OFFSET REG_BP

CALL CHECK_PREFIX           ; Check for "BP"

JZ   BASE_OK

MOV  DI, OFFSET REG_SI

CALL CHECK_PREFIX           ; Check for "SI"

JZ   BASE_OK

MOV  DI, OFFSET REG_DI

CALL CHECK_PREFIX           ; Check for "DI"

JZ   BASE_OK

MOV  AL, [SI]

CMP  AL, '0'

JB   INVALID_BASE

CMP  AL, '9'

JA   INVALID_BASE

MOV  DX, OFFSET MSG_DIRECT

CALL PRINT_STR

RET

INVALID_BASE:

MOV  DX, OFFSET MSG_INVALID

CALL PRINT_STR

RET

BASE_OK:

MOV  DX, OFFSET MSG_DIRECT

CALL PRINT_STR

RET

VALIDATE_ADDR ENDP

CHECK_PREFIX PROC

PUSH SI

PUSH DI

CP_LOOP:

MOV  AL, [DI]

CMP  AL, 0

JE   CP_MATCH               ; Reached end of DI string ? full prefix matched

MOV  BL, [SI]

CMP  AL, BL

JNE  CP_MISS                ; Mismatch found

INC  SI

INC  DI

JMP  CP_LOOP

CP_MATCH:

POP  DI

POP  SI

XOR  AX, AX                 ; AX=0 ? ZF=1 (match)

RET

CP_MISS:

POP  DI

POP  SI

OR   AX, 1                  ; AX?0 ? ZF=0 (no match)

RET

CHECK_PREFIX ENDP

STR_CMP PROC

SL: MOV  AL, [SI]

MOV  BL, [DI]

CMP  AL, BL

JNE  SN

CMP  AL, 0

JE   SE

INC  SI

INC  DI

JMP  SL

SE:

XOR  AX, AX

RET

SN:

OR   AX, 1

RET

STR_CMP ENDP

PRINT_STR PROC

MOV  AH, 09h

INT  21h

RET

PRINT_STR ENDP

INPUT_BUF   DB 30, 0, 31 DUP(0)

EXPR_BUF    DB 32 DUP(0)

INNER_BUF   DB 32 DUP(0)

KW_EXIT     DB 'EXIT', 0

REG_BX      DB 'BX', 0

REG_BP      DB 'BP', 0

REG_SI      DB 'SI', 0

REG_DI      DB 'DI', 0

CRLF        DB 0Dh, 0Ah, '$'

MSG_PROMPT  DB 0Dh,0Ah,'  Enter  : ','$'

MSG_DIRECT  DB '  Result: VALID.',0Dh,0Ah,'$'

MSG_INVALID DB '  Result: INVALID',0Dh,0Ah,'$'

MSG_BYE     DB 0Dh,0Ah,'Completed !!',0Dh,0Ah,'$'

END START

Module  3 : ASM_LINTER

ORG 100h

START:

MAIN:

MOV  DX, OFFSET MSG_PROMPT

CALL PRINT_STR

MOV  AH, 0Ah

MOV  DX, OFFSET INPUT_BUF

INT  21h

MOV  DX, OFFSET CRLF

CALL PRINT_STR

MOV  SI, OFFSET INPUT_BUF+1

XOR  CH, CH

MOV  CL, [SI]

INC  SI

MOV  DI, OFFSET LINE_BUF

JCXZ MAIN

UC_LOOP:

MOV  AL, [SI]

CMP  AL, 61h

JB   UC_OK2

CMP  AL, 7Ah

JA   UC_OK2

SUB  AL, 20h

UC_OK2:

MOV  [DI], AL

INC  SI

INC  DI

LOOP UC_LOOP

MOV  BYTE PTR [DI], 0

MOV  SI, OFFSET LINE_BUF

MOV  DI, OFFSET KW_EXIT

CALL STR_CMP

JZ   QUIT

CALL LINT_LINE

JMP  MAIN

QUIT:

MOV  DX, OFFSET MSG_BYE

CALL PRINT_STR

MOV  AH, 4Ch

INT  21h

LINT_LINE PROC

MOV  SI, OFFSET LINE_BUF

MOV  DI, OFFSET PAT_MOV_CS

CALL CHECK_PREFIX

JZ   ERR_MOV_CS

MOV  SI, OFFSET LINE_BUF

MOV  DI, OFFSET PAT_MOV_AX_AL

CALL CHECK_PREFIX

JZ   ERR_SIZE_MISM

MOV  SI, OFFSET LINE_BUF

CALL COUNT_BRACKETS

CMP  AL, 2

JGE  ERR_DOUBLE_MEM

MOV  SI, OFFSET LINE_BUF

MOV  DI, OFFSET PAT_PUSH_ONLY

CALL STR_CMP

JZ   ERR_PUSH_NO_OP

MOV  DX, OFFSET MSG_OK

CALL PRINT_STR

RET

ERR_MOV_CS:

MOV  DX, OFFSET MSG_ERR_CS

CALL PRINT_STR

RET

ERR_SIZE_MISM:

MOV  DX, OFFSET MSG_ERR_SZ

CALL PRINT_STR

RET

ERR_DOUBLE_MEM:

MOV  DX, OFFSET MSG_ERR_MEM

CALL PRINT_STR

RET

ERR_PUSH_NO_OP:

MOV  DX, OFFSET MSG_ERR_PUSH

CALL PRINT_STR

RET

LINT_LINE ENDP

COUNT_BRACKETS PROC

XOR  AL, AL

CB_L:

MOV  BL, [SI]

CMP  BL, 0

JE   CB_DONE

CMP  BL, '['

JNE  CB_NEXT

INC  AL

CB_NEXT:

INC  SI

JMP  CB_L

CB_DONE:

RET

COUNT_BRACKETS ENDP

CHECK_PREFIX PROC

PUSH SI

PUSH DI

CPX:

MOV  AL, [DI]

CMP  AL, 0

JE   CPX_OK

MOV  BL, [SI]

CMP  AL, BL

JNE  CPX_NO

INC  SI

INC  DI

JMP  CPX

CPX_OK:

POP DI

POP SI

XOR AX,AX

RET

CPX_NO:

POP DI

POP SI

OR  AX,1

RET

CHECK_PREFIX ENDP

STR_CMP PROC

SC:

MOV AL,[SI]

MOV BL,[DI]

CMP AL,BL

JNE SCN

CMP AL,0

JE  SCY

INC SI

INC DI

JMP SC

SCY:

XOR AX,AX

RET

SCN:

OR  AX,1

RET

STR_CMP ENDP

PRINT_STR PROC

MOV AH,09h

INT 21h

RET

PRINT_STR ENDP

INPUT_BUF     DB 40, 0, 41 DUP(0)

LINE_BUF      DB 42 DUP(0)

KW_EXIT       DB 'EXIT', 0

PAT_MOV_CS    DB 'MOV CS', 0

PAT_MOV_AX_AL DB 'MOV AX, AL', 0

PAT_PUSH_ONLY DB 'PUSH', 0

CRLF          DB 0Dh,0Ah,'$'

MSG_PROMPT    DB 0Dh,0Ah,'  Instruction: ','$'

MSG_OK        DB '  Result: OK - no error .',0Dh,0Ah,'$'

MSG_ERR_CS    DB '  ERROR : CS cannot be a destination register.',0Dh,0Ah,'$'

MSG_ERR_SZ    DB '  ERROR : Size mismatch - 16-bit dst, 8-bit src.',0Dh,0Ah,'$'

MSG_ERR_MEM   DB '  ERROR : MOV cannot have two memory operands.',0Dh,0Ah,'$'

MSG_ERR_PUSH  DB '  ERROR : PUSH requires an operand.',0Dh,0Ah,'$'

MSG_BYE       DB 0Dh,0Ah,' Finished!',0Dh,0Ah,'$'

END START

10. OUTPUT

- BUGS AND RECTIFICATION

- Multi Line Instruction :

- Multiple instructions were entered within one line, leading to wrong parsing, thus causing an address mismatch problem for the MOV command. This happened due to the requirement that only one instruction was supposed to be on each line .

- :

- Addressing mode module :

- As shown in the screenshot, the validation module for addressing mode works properly when valid data is entered and no errors are produced by the program. This was initially proven to check whether the module functions accordingly.

- Invalid usage of the address :

- The screen-shot provided is of an instance where there was an improper use of registers (SI and AX) within an addressing environment. The issue here is that the system could not identify the incorrectness in this regard. The revised screen-shot depicts a scenario where there is proper validation.

- Invalid usage of the address

- The snapshot represents the same erroneous data entry scenario after applying certain conditional checks. All such errors which earlier went unnoticed are now caught and reported through the program. The corrected snapshot is evidence that the condition has been applied effectively

- .

- Printing the whole Table :

- From the image above, it is clear that when a mnemonic or directive was typed, the program would wrongly display the entire list rather than just the corresponding item. This was because there was no proper way out of the loop. However, in the corrected screenshot below, only the matched instruction will be displayed.

- Printing the Mnemonics :

- The screenshot shows the revised behavior that prints out only the discovered mnemonic rather than listing several outputs. This was done through the inclusion of a jump instruction that stops the iteration process on the table once there is a hit

- .

- Invalid parameter in Tkinter:

- The screenshot below shows the error that occurred because of the use of a Tkinter function without having established proper linking of the module with an improper keyword parameter.

- Vertical Alignment Displacement Error :

- This glitch causes memory addresses on the 1 MB linear map to move and overlap, creating duplicated hex values such as 70000. There is no consistent vertical spacing because sometimes the rows will be cramped together while at other times the text will look ghosted or have corrupt characters such as A9?20C. This appears to be an internal user interface drawing problem because of the way that the gridlines are messed up.

- Skeleton view of the Final output :

The screenshot demonstrates the first GUI design implemented by using Tkinter that did not have any styling applied to it and had some memory problems. The memory problem was causing instability in the design and functionality of the GUI.

- Memory Map Panel Not Rendering (UI Component Failure)

However, when trying to delete the memory map panel, the right side does not display correctly.

The space where the panel should be is blank with some leftovers of UI elements, hinting at a problem with rendering.Other than that, the program operates fine, showing that the problem lies solely with the memory map panel.

- ASM Editor Interface Cleanup

The picture shows the x86 Assembly Editor with its user interface simplified, highlighting only the development space. This has been achieved by eliminating the memory map, which makes the programming interface more concentrated on coding and debugging.

12.CONCLUSION

The Assembly Aware Text Editor with Real-Time Analysis meets its objective of creating an intuitive and efficient IDE for the 8086 Assembly language code. The implementation of real-time analysis of syntax and semantics errors, dynamic syntax highlighting, auto-saved files, and context-based verification of commands and addresses makes the system very helpful in solving typical problems experienced by students and novices. The inclusion of a connection to the EMU8086 assembler greatly adds to the applicability of the system through its capability to assemble and run code in one click.

This project has helped in bridging the gap between theory and practice by introducing an easy way of learning programming on low-level languages. Although some issues like multi-line parsing and setting up Tkinter were faced, the end result is a stable system that provides valuable knowledge about Assembly programming. Possible improvements could be in the introduction of other architectures and debugger capabilities. This research will go a long way in computer science education by offering learners an easy-to-use environment for Assembly programming.